Caution: Risk of Shock.  Before Opening or Uninstalling the Computer System from Rack, ensure power has been removed from Power Supply(s) as applicable.

ESD Warning: Do not open the Computer System without proper ESD protective equipment and/or anti-static precautions in place.

Warning: As a result of this procedure, BIOS defaults will be loaded; any custom BIOS settings will need to be re-applied while returning the unit to service.  Please note any custom settings before proceeding.

- Battery Replacement

  - Battery location is dependent on your board model and form factor.  The cell is located in either a vertical or horizontal retention assembly.

  - Vertical Cell Holder Procedure:

    - Note the orientation of the battery.

Insertion 
Operation

Removal Operation

Grasp the battery between two fingers and gently pull upwards and away from the metallic negative contact.

    - Note the manufacturer of the cell in your board and replace it with an equivalent.

      - Insert new battery, negative side toward the open (metallic negative contact), positive side pointed away into the bottom of the Vertical Cell Holder.

      - Gently apply force towards the metallic negative contact until it latches into position.

    - Return the unit to service.

Removal of Vertical Battery

- Horizontal Cell Holder Procedure:

    - Note the orientation of the battery.

    - Depress the latching mechanism with a small implement and remove battery away from the latching mechanism.

    - Note the manufacturer of the cell in your board and replace it with an equivalent.

Remove from Horizontal Retention Assembly

Release Latching Mechanism

Insert new battery, positive side up, at a 45-degree angle on the side of the retention assembly opposite the Latching
Mechanism.

      - Gently press down on the Latching Mechanism side of the battery until it latches into position.

    - Return the Unit to Service.

Example of Horizontal Battery – 2

Example of Vertical Battery – 1

Trenton Systems Replacement CR2032 Battery PNs:

    - Maxcell:	 29-008017-000

    - Panasonic: 	 29-005774-000